HENCO Full flow manifolds:  Types VSKEK,

Fittings shall comply with EN-ISO 21003-2 and be approved by DVGW, KIWA, DNV, ATG suitable for the following applications:

- Drinking Water

- Heating

- Chilled water

- Other applications (on request and after HENCO’s written consent)

The modular full flow manifolds shall be manufactured from white PVDF,  and come in a 2- or 3-connection version with an alignment feature. Sealing O-rings shall be made of black EPDM material in accordance with EN681-1. Connections to the full flow manifold shall be made using PVDF or brass Eurocone fittings.

The full flow manifolds that uses the Push-Fit connections shall be compatible with Henco multilayer pipes to UNI EN 21003-2. Fittings shall have a maximum working pressure of 16 bar between temperatures of -10°C to +95°C.

The full flow manifold shall be installed and tested following the manufacturer’s installation instructions and according to appropriate codes of practice